RS-D620 Power Sequencer Requirement

8/17/2026

Takuya Katayama

Product Marketing

ASSP/Mainstream

Embedded Processor

Renesas Electronics Corporation

- Revision History

| Revision | Date | Author | Contents |
| --- | --- | --- | --- |
| 0.1 | 8/18/2026 | Takuya Katayama | Initial draft |

- Definition

  - Product

RS-D620 is based on RA6M5 with value added software for specific purposes, power sequencer. For time to market and cost optimization, there will be no silicon change. As it will be a separate product from RA6M5, documentation, web site, firmware, applications, tools, factory default flash data, etc. will be updated to have an updated branding.

  - Competitive Product

    - Texas Instruments UCD91320 / UCD90320 / UCD 91160 / UCD 90160

UCD 91320 is the latest power sequencer from Texas Instruments (TI) that leads power sequencer market. It updates from UCD90320 at lower prices and is expected to replace UCD90320. UCD91160 and UCD90160 are low-cost versions of UCD91320/UCD91160 with limited power line monitoring capability. TI provides Windows applications to configure power sequence processes and to review power sequencer situations.

UCD91320 data sheet, product information and support | TI.com

    - Analog Devices LTC2980-24

Analog Devices (ADI) LTC2980-24 is a secondary target product. While it supports less power rails than UCD91320, it claims higher accuracy in measurement. It also provides Windows application for configuration and needs to review for usability and functionality.

LTC2980-24 Datasheet and Product Info | Analog Devices

    - Renesas EU096

EU096 is RL78/GL12 based power sequencer development kit that engaged MXT for development. It monitors 6 rails with Windows configuration applications. It is not a competitive or target device but will be a good starting point for RS-D620 development.

EU096-PWRSEQPOCZ - Flexible MCU-based 6-Channel Power Sequencer Development Kit

MXT.ro – Nearshore partner for embedded products

    - Configuration Application

Both TI and ADI provide Windows applications for configuration updates, power sequencer feedback and error log decode. TI is consolidating and rebranding power sequencer applications to “UCD Sequencer Studio”. ADI provides “LTpowerPlay” for similar functionality. The leading customer explicitly requires ease of use for Windows applications and Renesas solution needs to exceed competitive products. In this documentation, the Windows application is named “Windows Configurator”

  - Target Market

    - AI Accelerator

While GPU was heavily used for AI training, AI accelerators designed to process specific neural network processing are becoming emerging AI accelerator design. All major CSPs are designing dedicated AI accelerators and ramping up with volume.

    - Enterprise Server (Adjacent)

Power sequencers are required for server CPUs such as Intel Xeon and AMD EPYC, and TI UCD series dominate the market. While RS-D620 would be a good fit for this market, at this moment, RS-D620 targets AI accelerator market as a primary target for time-to-market and makes enterprise server opportunities opportunistic. Possible future software update may be required to meet enterprise server requirements.

    - Network, Industrial and Medical Equipment (Adjacent)

Enterprise server CPUs are also used for network, industrial and medical equipment, which makes good fit for RS-D620; however, for the same reason to enterprise server, these markets are opportunistic.

  - Key Features

Following is the key feature that RS-D620 targets with comparison to competitive products

| Feature | Renesas RS-D620 | TI UCD91320 | ADI LTC2980-24 |
| --- | --- | --- | --- |
| Analog Rails | 22 or 26 | 24 | 24 |
| Digital Rails | 8+ | 8 | N/A |
| Margining | 20 | 16 |  |
| Sequencing | Supported | Supported | Supported |
| Cascading | 6 devices (132 or 156) | 4 devices (128) |  |
| OV/UV | Supported | Supported | Supported |
| Time-out | Supported | Supported |  |
| GPI input | Supported | Supported |  |
| Blackbox Fault Log | Supported | Supported | Supported |
| Single Event Upset | T.B.D. | Supported |  |
| Adaptive Voltage | 4+ Profiles | 4 Profiles |  |
| PMBus | PMBus 1.2, 1.3 or 1.4 | PMBus 1.2 | PMBus 1.2 |
| Package | T.B.D. | LQFP (14.0 x 14.0mm) nFBGA (9.0 x 9.0mm) | BGA (8.1 x 16.9mm) |
| Configurator | Windows Configurator | UCD Sequencer Studio | LTpowerPlay |

  - Customer Feedback

Following is the list of lead customer feedback that RS-D620 needs to satisfy.

- When over voltage is detected, the rail should be shut down < 1ms

- When over voltage is detected, all rails should be shut down < 5ms

- Shut down process may be simultaneous for all rails or sequential; need to be configurable

- ADC sampling frequency should be better than competitive products

- All pins are high at powerup and need to be pulled down.

- No OS preference

- Boot time target < 200ms

- Ball pitch cannot be 0.5mm; 0.8mm is preferred.

- Smaller package size is preferred, no form factor preference

- In general, data center requirement is 0 to 70 ambient temperatures

- Voltage measurement is required for most of rails, some rails may need currency information

- 10 rails with voltage and some currency measurements require up to 20 ADCs

- PWM based margining causes noise that goes over requirement; no PWM needed

- 2 cascading is required; 4 cascading is better.

- Cascading needs to be configurable for serial and parallel.

- Fault conditions must be shared across all cascaded units for simultaneous shutdown.

- No external memory

- No connectivity beyond I2C is required

- No specific PMBus version requirement; latest or latest -1

- No security is required; upper layer takes care of it.

  - POC

For further customer engagement, POC will be developed to demonstrate technical feasibility and following is the targeted feature set to emphasize.

- Windows application design and usability

- How to configure sequence is important

- "if" & "then" logic can be implemented via GUI

- There should be a time out option.

- Mechanism to read fault situation

- Cascaded environment should look like a single sequencer environment

  - Development Kit

For development, evaluation and demonstration, development kits may be required for the final product.

  - Schedule

Schedule is subject to change depends on customer schedule and engagement. Following is the current target schedule based on the leading customer product plan.

    - POC (Proof of Concept)

POC needs to be available by the end of Q3 ’26 to deliver customer demo in Oct. It does not need to be complete solution, and it focuses on Windows application with basic functionality on RA6M5

    - Function Complete (Beta)

The lead customer will start development in Q1 ’27 and beta version that supports all features need to be available by the end of Q4 ’26 (or early Q1 ’27)

    - Final

The lead customer plans to complete application in Q3 ’27 and final delivery should be aligned with it. Earlier delivery than customer schedule is preferred.

  - Future Consideration

There is similar product concept with EtherCAT being discussed with another customer. Implementation for D620 may be extended for it and Windows Configurator may need to support both like TI UCD Sequencer Studio supports multiple UCD products.

- Feature

  - Power Rail Monitoring

    - Digital Power Rail (Critical Fault)

Typically, power sequencer requires digital input for critical fault handling that does not go through ADC. Latency for this process needs to be extremely low. Power rail EN need to be disabled in a certain sequence or simultaneously. Windows Configurator needs to provide options.

POC verifies that ELC (Event Link Controller) can detect digital input pin signal and POE (Port Output Enable) can send out all 26 power rails immediately or can program timing sequence. Based on latency that POC can demonstrate, RS-D620 specification will be finalized. One of potential risks is MCU based solutions may have predictability challenges. If possible, POC should identify possible optimization and predictability risk toward productization.

    - Analog Power Rail

Monitoring will be handled by ADC and RS-D620 supports up to 26 channels with 2 ADC units. Scanner will be processed upfront continuously at 50MHz (every 20nsec) for 13 channels. UCD91320 sampling is 250nsec per channel and RS-D620 is on par.

OV, UV, temperature measurement, current measurement, sequencing time-out, PGOOD de-assert at runtime should be supported as fault detection. Then, log-only, retry N times then latch, shut down rail, shut down dependency group, full system shutdown in reverse sequence order should be supported.

POC should verify ADC can be achievable and measure latency to process it. To achieve lowest latency possible, analog comparator, ELC and POE need to be used. Based on POC, RS-D620 latency should be defined. Customer targets to detect over voltage and shutdown specific rail within 1msec, and all power rails in 5msec, following specific shutdown sequence defined by Windows Configurator.

RS-D620 has 26 ADC channels; however, there may be pin mux constraints. Through POC and specification review, maximum number of ADC channels available should be identified.

POC does not need to enable 26 ADC channels but needs to use reasonable numbers of ADC to estimate final product latency. It also does not need to support all fault conditions. In the meantime, it needs to demonstrate flexibility that “if” & “then” logic can be programmed in the Windows Configurator and reflected to RS-D620 behavior.

  - Sequence Logic

Built-in engine to configure Boolean logic equations, GPIO state machines, and Cascaded GPIOs to control complex board state changes need to be supported. As RS-D620 is MCU based power sequencer that provides flexible and conditional logic for sequencing, Windows Configurator should provide methods that developers can write code for sequencing conditions or GUI to design predefined logic with parameters.

For POC, it does not need to support full configuration, but need to demonstrate ease of use with cascaded environment.

  - Adaptive Voltage Identification

Adaptive Voltage Identification (VID) is an advanced power management technique where a processor dynamically requests unique operating voltages from a voltage regulator based on its specific manufacturing characteristics, real-time workload, and current temperature.

  - Margining

RS-D620 supports voltage margining, the process of deliberately forcing power supply rails to their upper and lower voltage limits. Limits can be configured by Windows Configurator and communicated to RS-D620 via PMBus. The device can perform accurate closed-loop margining on up to 20 rails using PWMs.

As the lead customer does not need margining, it is not required for POC.

  - Cascading

RS-D620 supports cascading up to 6 devices, which makes total of 132 (22/device) or 156 (26/device) depending on package selection. While the lead customer requirement is minimum 2 up to 4 cascading, cascaded competitive product supports up to 128 rails. RS-D620 has enough GPIOs to support additional cascading and target to support 6 cascading.

In the meantime, the lead customer requests “Cascaded environment should look like a single sequencer environment”. Windows Configurator should provide cascading settings to enable up to 6 devices and power sequence for all rails in 6 devices can be defined as if a single device. One of critical features for POC is usability of Windows Configurator and configuration for cascaded environment will be important.

  - Black Box Log (Event Logging)

Blackbox logging is a non-volatile (on-chip data flash) event recording features that capture critical system status and fault information during power failures or critical events. It preserves data across power dropouts to assist root-cause failure analysis. Event logging is Implemented using on-chip data flash (non-volatile logging). RS-D620 should capture data specifically for the first fault that occurs in the system and the last fault recorded right before a power-down event. It can be configured, cleared, and read out using Windows Configurator.

This is not required for POC.

  - PMBus

PMBus (Power Management Bus) is an open-standard digital power management protocol used to configure, control, and monitor power supplies and converters in electronic systems. It is based on the I2C physical interface and extends the System Management Bus (SMBus) protocol. RS-D620 is based on RA6M5, which supports I2C and SMBus, and it needs to extend for PMBus. RS-D620 and Windows Configurator need to support PMBus via USB to I2C bridge. PMBus 1.4 support is nice to have, PMBus 1.2 is minimum requirement. Requirement of PMBus validation is to be finalized through customer engagement.

For POC, PMBus is not required; but for Windows Configurator concept, limited or complete PMBus implementation may be needed.

  - JTAG

JTAG (Joint Test Action Group) is a widely used industry standard (IEEE 1149.1) that provides a dedicated physical interface for testing, debugging, and programming integrated circuits (ICs) on a circuit board. It is enabled for RS-D620 configuration

  - USB

USB (Universal Serial Bus) is the industry-standard interface for short-distance digital data communications and electrical power delivery. While the USB hardware peripheral handles serialization on the wire, the software stack and interrupt service routines (ISRs) introduce latency overhead that can delay other critical real-time tasks. RS-D620 needs to achieve predictable behavior as much as possible and USB should be disabled.

  - Connectivity

SCI (Serial Communication Interface), SPI/QSPI/OSPI (Serial Peripheral Interface), CAN-FD (CAN with Flexible Data rate), SDHI (SD/MMC Host Interface), Ethernet MAC, CEC (Consumer Electronics Control module) also need to be disabled for lower latency and to free up GPIO pins

  - Watchdog & System Reset

Watchdog & system reset should be implemented using internal watchdog and GPIO.

This is not required for POC.

  - Dual Bank Safe Operation

Dual-bank fail-safe operation should be supported. While the device is actively sequencing, trimming, and monitoring rails using its current configuration, Windows Configurator can download a new configuration script via the PMBus interface.

This is not required for POC.

  - Single Event Upset (SEU)

UCD91320 supports single event upset (SEU) detection using ECC memory. It rarely happens and LTC2980 does not have the equivalent feature. RS-D620 supports ECC memory, the first 64KB of SRAM is ECC protected, the following 448KB is parity protected; however, it is not clear if this is MUST have feature yet. SEU support will be defined through further customer engagement.

This is not required for POC.

  - Initialization

    - Initialization

While the lead customer does not have specific requirements on hardware initialization, there seems to be some customers looking for minimum initialization time. Renesas has earlier exploration to optimize hardware initialization using RA4T1 platform and similar approach should be applied to RS-D620.

Internal POR circuit releases internal reset 4.5 ms after VCC reaching threshold. Reset release can be done faster by using external sequencing for RES pin reset. External sequencing must release #reset after 0.7 ms of VCC reaching threshold, which complete peripheral initialization in 1.0msec.

This is not required for POC.

    - Boot

The lead customer does not have OS preference and boot time requirements; but prefer faster boot. Tentatively it is agreed to target OS boot to be completed within 200msec. For RS-D620, there is no specific OS requirement; however, there is possibility that a part of component needs to be reused for RS-D920, which requires Zephyr, current target is Zephyr for RS-D620.

Boot time optimization is not required for POC.

    - Product Detection

As RS-D620 will be physically same as RA6M5, which may be priced differently, we would like to protect customers from purchasing RA6M5. Windows Configurator and potentially firmware that enable power sequencer unique features need to detect product name from factory flash so that only it works for RS-D620. Factory Flash Part Numbering Register at address 0x0100_80F0

This is not required for POC.

- Silicon Configuration

  - Interfaces

To differentiate from competitive products that pin mapping is based on power sequencer definition, RS-D620 expects to provide best flexibility to customers with physically maximum number of GPIO for information gathering. It does not need to physically disable non-GPIO use cases; but need to clarify maximum number of GPIO, ADC, PWM based on best case mux.

  - Product Name

Factory Flash Part Numbering Register at address 0x0100_80F0 should be referenced so that Windows Configurator and potentially firmware can detect RS-D620 from RA6M5.

This is not required for POC

- Windows Configurator

UCD Power Sequencer Studio is the target implementation and all functionality it support needs to be implemented in Windows Configurator.

  - Rail Setup & Sequencing Configuration

Following should be implemented.

- Timing & Dependencies: Drag-and-drop tools to program Turn-On and Turn-Off delays, ramp times, and interlocking logic for up to 32 power rails

- System Logic & GPIOs: Built-in engine to configure Boolean logic equations, GPIO state machines, and Cascaded GPIOs to control complex board state changes

- Logic Coding: RS-D620 is MCU based power sequencer that provides flexible and conditional logic for sequencing. Windows Configurator should provide methods that developers can write code for sequencing conditions or GUI to design predefined logic with parameters.

- Multi-Device Cascading: Synchronized multi-chip interfaces that allow hardware designers to daisy-chain up to 6 hardware sequencers to control a massive matrix of up to 156 hardware rails

  - Telemetry, Monitoring, and Calibration

Following should be implemented.

- Real-time Oscilloscope View: Live graphical charting tools that visualize voltage, current, and temperature telemetry directly from the device's ADC channels

- Analog & Digital Monitoring: Configures thresholds for Overvoltage (OV), Undervoltage (UV), Overcurrent (OC), and Timeout conditions.

- Closed-Loop Margining & Active Trim: Features a dedicated interface to push regulators to high/low limits for stress testing, alongside an Active Trim module to calibrate and optimize regulator output voltages

  - Diagnostics & System Safety Management

- Black Box Fault Logging: Decodes the nonvolatile hardware "Black Box" log, letting you download timestamps from the Real-Time Clock (RTC) alongside the specific status profiles of all rails during the exact moment a failure occurred

- System Safeguard Provisioning: Directly configures internal system parameters such as the Software Watchdog Timer (SYSTEM_WATCHDOG_CONFIG) and System Reset Actions (SYSTEM_RESET_CONFIG) via standard GUI menus.

- Status Register Analytics: Displays an organized breakdown of all standard PMBus status flags, allowing engineers to determine if a fault was triggered by temperature, a logic input, or a voltage spike

  - Code Generation & Register Access

- PMBus Register Map Interface: Offers a low-level memory viewer to inspect or change individual PMBus operations and standard commands directly.

- USB-to-GPIO: Windows Configurator communicate with RS-D620 via USB-to-GPIO bridge

- Configuration Exporting: If there is any information on RS-D620 needs to be reviewed, the Windows Configurator will be the platform to retrieve information from the device.

- Package

  - Parts

Following is current candidate package configurations that have 26 ADCs, 133 GPIO and 176 pins. Package selection is subject to change based on customer feedback.

| Part Number | Package | Pkg. Size (mm) |
| --- | --- | --- |
| R7FA6M5AH3CFC#BA0 | LFQFP | 24 x 24 x 1.7 |
| R7FA6M5AH2CBG#BC0 | FBGA | 13 x 13 x 1.4 |

  - Pin Assignment

When 176 pin package is selected, following shows pin assignment. There may be some mux constraints and actual GPIO, ADC, PWM availability is to be reviewed.

| JTAG |  | 4 |  |
| --- | --- | --- | --- |
| PMBus |  | 7 |  |
| Power Rail 1 | EN (GPIO) | 1 | 52 + up to 20 (# or margining up to 20) |
|  | Voltage Monitoring (ADC) | 1 |  |
|  | Margining (PWM) | 0 or 1 |  |
| Power Rail 2 | EN (GPIO) | 1 |  |
|  | Voltage Monitoring (ADC) | 1 |  |
|  | Margining (PWM) | 0 or 1 |  |
| … |  |  |  |
| Power Rail 26 | EN (GPIO) | 1 |  |
|  | Voltage Monitoring (ADC) | 1 |  |
|  | Margining (PWM) | 0 or 1 |  |
| Cascading | Cascading 1 | 1 | Up to 7 |
|  | Cascading 2 | 1 |  |
|  | … |  |  |
|  | Cascading 6 | 1 |  |
|  | SYNC_CLK | 1 |  |
| Host Communication | WDI | 1 | 7 |
|  | WDO | 1 |  |
|  | Power | 1 |  |
|  | Good | 1 |  |
|  | Warning | 1 |  |
|  | System | 1 |  |
|  | Reset | 1 |  |
| Unassigned |  | 36 |  |
| Total |  | 133 |  |